A Microprocessor-Controlled SWR/Power/Return Loss Monitor with Advanced Features

By: Larry Coyle, K1QW

_____________________________________________________________________

This instrument uses a unique microprocessor to drive an analog panel meter, exploiting the best features of both analog and digital technology.

_____________________________________________________________________

Even though we're well into the digital age, there are places where the old standby analog approach still seems to be exactly right. When you’re trying to find a peak or a dip while tuning up your rig or if you’re just monitoring a signal level that's constantly changing, a traditional panel meter with a real moving pointer can be just what the doctor prescribed. And I’m sure there are times when you’d rather flip a few switches instead of stepping through layer after layer of digitally-inspired menus.

On the other hand, digital technology has its place, too. One of the many facets of ham radio which I enjoy is designing hardware using microprocessors for a performance boost. This, along with the need for a nice SWR monitor for my shack, prompted me to develop an instrument using the best of both worlds – analog for capturing data and displaying the results, digital for all the heavy number crunching in between.

Perhaps, like me, you disfavor older SWR monitors which use a crossed-needle arrangement to indicate SWR (confusing to interpret) and display transmitted power on a non-linear scale (misleading, at a quick glance). If I could get around these liabilities by drawing on the power of digital processing,  and go on to include a few features not normally found in a homebrew design – well,  what's not to like?

Overview and Block Diagram

After a bit of work, I came up with the SWR/Power/Return Loss monitor shown in block diagram form in Figure 1.  The three interacting components that make up the instrument are: SWR sense head, front panel assembly and signal processor board. (The latter two make up the Display and Signal Processor unit.) The SWR sense head is a conventional Stockton dual-transformer bridge with Schottky diode detectors. This component converts the forward and reflected RF amplitudes to dc and connects to the Display and Signal Processor by an ordinary stereo audio cable.

The user interface to the SWR monitor is familiar and intuitive. As you can see in Figure 2, a front-panel rotary switch selects one of several quantities to be measured and an analog meter displays the result. The rear panel, Figure 3, shows the power switch and connections to the outside world. There are no multi-function buttons leading to a maze of menus within menus. A glance at the controls and it's obvious how to use them.

At a turn of the rotary switch, the meter can display:

- Forward power on a linear scale

- Reflected power as a percentage of forward power

- Net power delivered to the load

- SWR on a linear scale (no crossed needles to interpret)

- Return loss on a linear db scale

If you're not familiar with the term "return loss", you should know that at bottom it's just a way to represent impedance mismatch. So is SWR. However, sometimes return loss is a lot more convenient.  For more insight, look at the sidebar included in this article.

Some additional features which distinguish this instrument are:

- Peak hold front panel switch

- A front panel switch to multiply the meter sensitivity by 3X

- Data logging to a computer using the USB port.

- Transmitter Lockout (XLO) at high SWR

Transmitter Lockout (XLO) is a protective feature that lights a LED on the front panel and energizes a relay when the SWR exceeds a preset level. This opens a set of relay contacts which are connected to a pair of banana jacks on the rear panel. If you run your transmitter’s PTT or keying line through these jacks, the transmitter will not turn on at high SWR, ideally saving the power amplifier from damage.  Once triggered, the relay remains energized for either three or nine seconds, depending on the configuration of the DIP switch located on the signal processor board. The same DIP switch also allows you to select the SWR level where the XLO relay triggers – 1.5, 2.0, 3.0 or 4.0.

The two dc signals from the sense head make up the analog input to the signal processor board. The analog output for driving the meter is a pulse width modulated (PWM) signal generated by the microprocessor and smoothed by a simple resistor-capacitor (RC) filter.  (As a side benefit, the meter movement cannot be damaged by an overload, since it’s protected by software.) Between analog input and output, however, it's all digital, courtesy of a rather unusual microprocessor chip.

The Digital Workhorse Chip

The microprocessor I settled on is made by Parallax, Inc. and is truly unique. I had been eyeing it with interest for some time and only needed a good application to come along for me to jump in and get familiar with it. This SWR monitor project seemed like a perfect fit.

The outstanding feature of this chip is that it’s really eight processors in one package, each one able to run its own separate program. The designers at Parallax have dubbed this chip the "Propeller", and use the analogy of a central hub which spins around to service each of the eight peripheral processors (also known as "cogs") in turn, giving each one its own time slot to execute its program.

This may sound more than a little challenging, especially if you're not familiar with microcomputers, but this architecture actually makes programming the chip much easier. If you need to carry out several tasks at once -- and most computer applications do – you can just assign each job to its own individual cog and work on them separately. The central hub makes sure each cog gets its turn and keeps things synchronized.

The icing on the cake is the Propeller USB Development Board, manufactured by Parallax. This 3 x 4 inch printed circuit board is available from several suppliers and has factory-installed the Propeller chip itself, an Electrically Erasable Programmable Read-Only Memory (EEPROM) for program storage, a 5 MHz crystal, two voltage regulators and, best of all, a USB mini-B connector and all the necessary USB interface circuitry.

Parallax has made the Propeller chip extremely easy to use. For one thing, everything you need to write software for and to program the Propeller is available for free on the Parallax web site. There's also an extensive library of pre-built code snippets which you can just drop into your own programs. The Propeller is programmed in a language called "Spin" which should make anyone who is familiar with the BASIC computer language feel at home.

Software is loaded onto the development board using the aforementioned USB port. All you need is a host computer with USB support and the free software development tools available on the Parallax web site. These tools take care of all the details of compiling your Spin-language source code and downloading to the Propeller chip. When not in use as a programming port, the same USB connection is used as a serial data port for passing data between the SWR monitor and a computer running a terminal program such as HyperTerminal.

Processor Board Circuit Details

The development board includes a prototyping area large enough to hold all the parts necessary to complete the SWR monitor project, except for the front and rear panel components. Since most of the parts are already in place, the construction of the digital board is quite easy. As you can see in Figure 4, the electrical schematic, only two integrated circuits and a few analog components are required.

The forward and reflected levels converted to dc in the SWR sense head are received by dual op-amp U2. Each section of U2 is configured as a unity-gain voltage follower with a compensating diode (D1 and D2) to balance out the dc offset introduced by the Schottky rectifier diodes in the sensing head. Each section also has an RC low pass filter at its input to help fight off unwanted RF -- always a possibility around a ham shack.

The next stage in the signal path is U3, an analog multiplexer (MUX). This chip acts as a three-position switch under control of the microprocessor. (There is a fourth channel, which is unused.) The forward and reflected signals (on pins 14 and 15) are sampled alternately, 20 times per second, as long as the program is running. The third input to the MUX (pin 12) is grounded, and is also sampled by the microprocessor from time to time. This gives a measure of any dc offsets introduced by the analog-to-digital conversion and is used to correct the forward and reflected readings in software.  This automatic zero adjustment takes place every five seconds or so and the “Calibrating” LED on the front panel flashes briefly whenever this happens. It also serves as a digital heartbeat indicating that the software is running through all of its routines.

The MUX output (pin 13) is passed to a potentiometer voltage divider to allow the scale factor to be set during calibration. From there it goes to the Propeller microprocessor itself.

You may ask, “Why are we feeding an analog signal into a digital chip?” The answer is that the Propeller, as do most microprocessors, has the ability to perform an analog-to-digital conversion (ADC) using only a couple of digital I/O pins and a few passive components. It accomplishes this feat by periodically pumping charge into a capacitor, effectively integrating it. This capacitor is tied to the voltage to be measured through a resistor. The microprocessor keeps count of how many charge packets are needed to balance the resulting voltage against a reference, so it has a digital indication of the input voltage level. This technique doesn't achieve the accuracy or the speed of high-performance ADC chips, but it's cheap, convenient and plenty good enough for our purposes.

Digital Ins and Outs

The microprocessor responds to the user's inputs by monitoring the state of a number of input pins (thirteen, to be exact). Most of these pins are controlled by the rotary and toggle switches on the front panel, which connects to the processor board by a short ribbon cable. See Figure 5 for a schematic of the separate front panel assembly. This cable also carries power for the LEDs. Back on the processor board there are a multi-section Dual In-line Package (DIP) switch, S2, and two jumpers, JP1 and JP2, which select among the various operating modes. Lastly, there's the Xmtr Lockout (XLO) switch, mounted on the rear panel and visible in Figure 3.

These thirteen inputs to the microprocessor and the various modes of operation they control are summarized in the table of Figure 6.

The list of digital outputs is much shorter. There are only six:

- Two address lines for the analog MUX, to select among the three 	input channels

- Two lines to power the Calibrating and XLO LEDs located on the front 	panel

- One PWM signal to drive the analog meter on the front panel

- One line to control the XLO relay.

Because of the generous prototyping area, there is plenty of room to mount the required components to complete the circuit. Most of the room on the prototype board is taken up by the XLO relay and the DIP switch array used to set various operating modes. Figure 7 is a photo of the built-up processor board nestled inside the 2 x 4 x 6 inch aluminum enclosure. Here you can also see some of the wiring to the rear panel components. The USB connector is at the top edge of the circuit board and is accessible through a rectangular hole nibbled out of the top surface of the aluminum box.  Figure 8 is a picture of the entire display and data processing unit with the front cover moved aside to show the ribbon cable connecting the processor board and the front panel assembly.

The Sense Head

The SWR sense head is the business end of the SWR meter. Here is where the forward and reflected amplitude components of the RF present on the transmission line are separated out and converted to dc. For this project I adopted a straightforward Stockton directional coupler circuit, with Schottky diode detectors. I particularly like this design because it’s easy to build and, unlike other designs, requires no balancing trimmers. The schematic is given in Figure 9 and a photo of the assembled unit (built in the traditional Altoids® tin) is shown in Figure 10. This article will not go into further details of this circuit, since it has been described in many amateur radio publications over the years.

A few additional notes, however: The accuracy of this circuit can be greatly improved by taking care to match the diodes in the sensing head with the corresponding compensation diodes in the op amp input stages on the signal processor board (D1 and D2 in Figure 4). The easiest way to do this is with the diode test function included on most utility digital multi-meters which passes a small dc current through the diode being tested and displays the forward voltage drop.

It's also worth noting that the high-frequency range can be extended a bit by using surface-mount components in the sensing head. An early design I built using leaded components showed an SWR error of 13 percent in the six meter band. By going to surface mount components I was able to reduce the error to less than five percent.

I built up two versions of the SWR sensing head. One is suitable for QRP use, having a maximum range of 10 watts. The other can handle up to 100 watts. The only difference between the two versions is the number of turns on the toroid transformers (See the parts list in Figure 9). The high frequency performance of the 100-watt head at six meters proved to be poorer than the 10-watt unit. I attribute this to the added capacitance of the transformer windings (the higher power transformers have three times as many turns as the low power ones).

The overall accuracy of the monitor compares fairly well to commercial devices. Figure 11 (A) and (B) shows performance data I took using dummy loads of 50 and 96.7 ohms. At five watts input, using the 10-watt sensing head, the SWR readings checked out within three percent over a range of 1.8 to 30 MHz, and better than five percent at 50 MHz. Part (B) of the figure shows good accuracy down to 500 milliwatts input (great for QRP rigs!) and is even usable down to 250 milliwatts.

Software

The software for the SWR/Power/Return Loss meter was written entirely in Propeller Spin language. I will only outline the software in broad terms here. For those who want to duplicate this project or are just interested in the inside details, the Spin language software listing, ready to program into the Parallax USB Development Board, is available on my web site   as well as the ARRL web site [***  EDITOR: Please Insert Note  ***]. There are also details of the subroutines handled by each of the Propeller cogs plus additional text and graphic files covering operation and features of the instrument.

As explained above, the organization of the Propeller into several peripheral processors (cogs) makes it easy to follow the program flow.

The main program object (MAIN) runs in its own cog and is the first to be executed when power is applied (or whenever the chip is reset). Its first order of business is to set individual microprocessor pins to be inputs or outputs, as required.

Next, MAIN starts up three separate cogs for (1) analog-to-digital conversion (the ADC3CH cog), (2) generating the pulse width modulated waveform to drive the analog panel meter (the PWM cog) and (3) communicating with the terminal via the USB port (the MONITOR cog). They run continuously, swapping data with the MAIN program by means of variables stored in memory. After these three cogs are started, the MAIN program object enters a loop, endlessly repeating the following steps:

- Read forward and reflected amplitudes and SWR data generated by 	the ADC3CH object.

- Calculate forward power, percent reflected power, net power, SWR 	and return loss in decibels.

- Display the quantity selected by the front panel switch on the 	panel meter using the PWM object.

- Convert power levels, SWR and return loss information to ASCII 	strings and 	send them to the MONITOR object, which writes to the 	USB port.

I was able to make good use of the free Propeller software library for the analog-to-digital and data conversion routines and for the computer terminal interface. These alone saved me much effort in developing the software; but what saved the most work of all were the extensive math packages that make decibel and floating-point conversions a snap.

This is a shining example of building on the accomplishments of others who have made their work freely available. Without such an extensive library of pre-tested routines on hand, I probably would never have even started this project.

Concluding remarks

The SWR/Power/Return Loss monitor is still a work in progress. I would like to build up a VHF/UHF sensing head to extend the frequency coverage. Since there is still plenty of room in the Propeller’s memory, I may add some more features. Perhaps an audible indication of SWR level?

This was an enjoyable and interesting project. It resulted in an instrument useful around the shack and also gave me the chance to get familiar with an unusual microprocessor.  I feel sure it will prove useful on other projects I have in mind.

I hope that this article may spark an interest in some hams to blend the fine old traditions of amateur radio with the innovations of electronic hobbyists who build projects with embedded microprocessors. There's excitement, fun and a great deal of satisfaction to be found in both of these arenas.

Biographical sketch

Larry Coyle, K1QW, was first licensed in the 1950s when he was in high school. His professional career began as a radio broadcast engineer, and after graduating from college with an MSEE degree, he spent the next four decades designing electronics systems for military and aerospace applications. After running a free-lance consulting practice for several more years, he retired. His enthusiasm for ham radio never faded away, and he is now the holder of an Extra Class ticket and a member of the Wellesley (MA) Amateur Radio Society. You may catch him on any HF band operating QRP mostly in CW and PSK31 modes. You can also reach him at 100 Rolling Lane, Needham, MA  02492 – or at lmcoyle1@verizon.net .

Parallax, Inc., 599 Menlo Drive, Rocklin, California 95765, www.parallax.com

The technique of using a diode in the feedback path of an op amp to compensate for the non-linearity of an RF detector diode is described in detail in “A Compensated, Modular RF Voltmeter”, ARRL Handbook for Radio Communications, 2007, pp 25.35-25.41.

This approach to A-to-D conversion is known as Delta-Sigma conversion and is widely used in the design of integrated circuits. This technique is capable of great accuracy if the reference level is clean and precise. In our case, we are using the logic switching level of the microprocessor input pin as a reference, so the accuracy is somewhat compromised. But it’s still enough for this design.

J. Grebenkemper, KA3BLO, “The Tandem Match – an Accurate Directional Wattmeter”, QST, Jan 1987, pp 18-26. This circuit has also appeared in several editions of the ARRL Handbook as well as the ARRL Antenna Book.

www.lcbsystems.com