There are some component additions to the BOT Instructor’s Board if you intend to use in the Mars Exploration simulation.  Some of the components are part of the basic BOE-BOT parts kit that came with your robot, other part need to be either purchased or obtained through a grant from the ARRL Education and Technology Progress grant.

Install the button speaker that comes with the BOE-BOT at the indicated location.  Take note of the speaker polarity (if it is marked).

Install the photo resistor (sensor) that comes with the BOE-BOT at the indicated location.  The component does not have polarity.

Install the temperature sensor IC DS1620 at the indicated socket on the board.  This component can be purchase or is part of the grant.  Note the notice at one end of the IC.  Note that the IC socked also has a notch.  Match the notches before you insert the IC in the socket.

Install the compass module on the board at the indicated location.   The compass module can be purchased or is part of the grant.  The compass module is a small circuit board.  You will note there is a small rectangular component on one end of the module, mount the compass module so that this small rectangular component is toward the word “compass” that is printed on the board.  The module should be mounted level to the board surface, and slightly elevated (about 1/8th of an inch).  Solder the compass module in place from the bottom of the BOT Instructor’s Board

Install the data link transmitter at the indicated location on the board.  The transmitter board has 4-pins that are inserted into the appropriate socket that also has 4-pin receptacle.  The 5-pin socket can be used if in the future you wish to put the receiver on the BOT Instructor’s Board.  Install a screw from the bottom side of the board to hold the transmitter in place.

The data link receiver portion of the system comes with the grant.  It is a complete tested board and should be ready to go.  You will need to install on your computer the Excel Spreadsheet to display the data during the simulation.  This spreadsheet is a modification of the StampDAQ program that is included on the CD-ROM and also available from the Parallax web page.  You will need to study how this program works with the data link.

This completes the addition of components to the BOT Instructor’s Board for the Mars Lander simulation.  Complete the compass calibration as detailed in the documentation that came with the compass module.