Sidebar - Return Loss

Most hams are used to dealing with SWR readings as an indication of impedance match. There's nothing wrong with that familiar approach, but sometimes a measurement of “return loss” is easier and more informative. Think of return loss this way: If you send a signal from a transmitter or any generator out to a load, some part of it -- the part that's not absorbed by the load or dissipated along the way -- will be reflected back to you. The part that’s reflected back to the source is “lost” by the load.  Divide the amplitude (voltage or current, it doesn’t matter) of the reflected RF wave by the amplitude you transmitted and you have what is known as the "reflection coefficient". Express the magnitude of the reflection coefficient in decibels and you have return loss.

In the more concise language of mathematics, it looks like:

Reflected voltage

Reflection coefficient = -----------------------

Transmitted voltage

and,

Return Loss = -20 log(Reflection Coefficient)

Or, if we are dealing with reflected and transmitted powers, rather than voltages,

(    Reflected Power       )

Return Loss = -10 log(----------------------------)

(   Transmitted Power   )

The minus sign in the return loss formulas is just a mathematical trick to make the answer come out to be a positive number. This is standard practice in the RF engineering field.

For a deeper understanding of this subject, you might want to read the excellent "Hands-On Radio" column by Ward Silver, N0AX, which appeared on page 76 of the January 2009 issue of QST.

Ideally, you would like nearly all of the transmitted energy to be absorbed by the load and practically none of it to be reflected. This situation represents a very small reflection coefficient (much less than 1) and a high return loss. You will also recognize this condition as an SWR close to 1.0. Generally speaking, a high value of return loss is a desirable thing, and indicates a low SWR level.

A return loss reading of 30 db, for instance, is very good. This equates to a SWR of about 1.06, and is much easier to read on the meter.

Checking return loss can give you a better perspective on your antenna system's performance. For example, an SWR reading of 2:1 might send you scrambling to tune up your system because of the "horrible" SWR. But this SWR is the same as a return loss of 9.5 db, meaning that only 11% of your transmitted power is being “lost” by the load (by being reflected back to the source) and 89% is absorbed by the load. Looked at this way, it doesn't sound all that bad.

The return loss measurement capability of this monitor can be used to check the degree of mismatch of filters, matching stubs, antennas, power amplifier inputs and almost any 50 ohm device that can handle a power level of 500 milliwatts. Coax cable loss is also easily measured by the return loss method -- very handy if you need to check out an old piece of coax which may have deteriorated over time.

There are some corrections to the information published in this column. See the “Feedback” note in February 2009 QST, page 53.