| Digital Input | Originates at | What it does |
| --- | --- | --- |
| X3SEL | front panel - toggle switch | increases meter sensitivity by 3X |
| PEAKENBL | front panel - toggle switch | enables peak hold function on the meter |
| RLOSSEL | front panel – rotary switch | displays return loss on the meter |
| SWRSEL | front panel – rotary switch | displays SWR on meter |
| NETSEL | front panel – rotary switch | displays net power on meter |
| REFSEL | front panel – rotary switch | displays percent reflected power on meter |
| FWDSEL | front panel – rotary switch | displays forward power on meter |
| HIREFSEL | proc board - DIP switch | drives meter to full scale for calibration (must be OFF for normal operation) |
| FASTCAL | proc board - DIP switch | speeds up meter response for calibration (must be OFF for normal operation) |
| XLOLONG | proc board - DIP switch | enables long xmtr lockout ( about 9 seconds) |
| XLTHRESH0 | proc board - DIP switch | XLO threshold bit 0 |
| XLTHRESH1 | proc board - DIP switch | XLO threshold bit 1 |
| XLOCKENBL | rear panel switch | enables xmtr lockout function |
| DIAG | proc board - jumper JP1 | displays raw input levels on terminal |
| W100 | proc board - jumper JP2 | changes terminal display to 0-100 watt range (required when using 100-watt sense head) |

XLTHRESH1 and XLTHRESH0 form a two-bit code to select one of four SWR levels. When this SWR level is exceeded and if XLO is enabled, the xmtr lockout function is triggered.

| XLTHRESH1 | XLTHRESH0 | Selects SWR threshold level |
| --- | --- | --- |
| OFF | OFF | 1.5 |
| OFF | ON | 2.0 |
| ON | OFF | 3.0 |
| ON | ON | 4.0 |

Figure 6 – Digital inputs to microprocessor chip