### Lines and refresh rate

The NTSC format is used with the M format (see broadcast television systems), which consists of 29.97 interlaced frames of video per second. Each frame consists of 486 visible scanlines out of a total of 525 (the rest are used for sync, vertical retrace, and other data such as captioning). PAL uses 625 lines, and so has a higher picture resolution. The NTSC system interlaces its scanlines, with visible scanlines 21-263 drawn in the first field, and visible scanlines 283-525 drawn in the second field, yielding a nearly flicker-free image at its approximately 59.94 hertz (nominally 60 Hz/100.1%) refresh frequency. The refresh compares favorably to the 50 Hz refresh rate of the PAL and SECAM video formats used in Europe, where 50 Hz alternating current is the standard; flicker was more likely to be noticed when using these standards until modern PAL TV sets began using 100 Hz refresh rate to eliminate flicker[citation needed]. This produces a far more stable picture than native NTSC and PAL had, effectively displaying each frame twice. This did, at first, cause some motion problems, so it was not universally adopted until a few years ago. Interlacing the picture does complicate editing video, but this is true of all interlaced video formats, including PAL and SECAM.

The NTSC refresh frequency was originally exactly 60 Hz in the black and white system, chosen because it matched the nominal 60 Hz frequency of alternating current power used in the United States. Matching the screen refresh rate to the power source avoided wave interference that produces rolling bars on the screen. Synchronization of the refresh rate to the power cycle also helped kinescope cameras record early live television broadcasts, as it was very simple to synchronize a film camera to capture one frame of video on each film frame by using the alternating current frequency as a shutter trigger.

The figure of 525 lines was chosen as a consequence of the limitations of the vacuum-tube-based technologies of the day. In early TV systems, a master voltage-controlled oscillator was run at twice the horizontal line frequency, and this frequency was divided down by the number of lines used (in this case 525) to give the field frequency (60 Hz in this case). This frequency was then compared with the 60 Hz power-line frequency and any discrepancy corrected by adjusting the frequency of the master oscillator.

The only practical method of frequency division available at the time was the use of multivibrators, which could only divide by small numbers. For interlaced scanning an odd number of lines per frame was required, and so a chain of multivibrators was needed, each of which had to divide by a small, odd number. (Note that an odd number is never divisible by any even number). The closest practical sequence to 500 was 3 × 5 × 5 × 7 = 525. Similarly, the British 405-line system used 3 × 9 × 9 × 5. Although other values were theoretically possible, all of them involved division by unacceptably large numbers like 13 or 17, which produced reliability problems. Modern systems derive all their frequencies from the color subcarrier frequency (see below).

In the color system the refresh frequency was shifted slightly downward to 59.94 Hz to eliminate stationary dot patterns in the color carrier, as explained below in "Color encoding".

Color encoding

There are 3 main standards in use around the world, PAL (Phase Alternating Line), NTSC (National Television System Committee) and SECAM (Séquentiel Couleur à Mémoire — Sequential Color with Memory).

The system used in North America is NTSC. Western Europe, Australia and Eastern South America use PAL. Eastern Europe and France use SECAM. Generally, a device (such as a television) can only read or display video encoded to a standard that the device is designed to support; otherwise, the source must be converted (such as when European programs are broadcast in mainland North America or vice versa).

This table illustrates the differences:

|  | NTSC M | PAL B,G,H | PAL N | PAL M | SECAM B,G,H | SECAM D,K,K',L |
| --- | --- | --- | --- | --- | --- | --- |
| Lines/Fields | 525/60 | 625/50 | 625/50 | 525/60 | 625/50 | 625/50 |
| Horizontal Frequency | 15.734 kHz | 15.625 kHz | 15.625 kHz | 15.750 kHz | 15.625 kHz | 15.625 kHz |
| Vertical Frequency | 60 Hz | 50 Hz | 50 Hz | 60 Hz | 50 Hz | 50 Hz |
| Color Subcarrier Frequency | 3.579545 MHz | 4.433618 MHz | 3.582056 MHz | 3.575611 MHz |  |  |
| Video Bandwidth | 4.2 MHz | 5.0 MHz | 4.2 MHz | 4.2 MHz | 5.0 MHz | 6.0 MHz |
| Sound Carrier | 4.5 MHz | 5.5 MHz | 4.5 MHz | 4.5 MHz | 5.5 MHz | 6.5 MHz |

For backward compatibility with black and white television, NTSC uses a luminance-chrominance encoding system invented in 1938 by Georges Valensi. Luminance (derived mathematically from the composite color signal) takes the place of the original monochrome signal. Chrominance carries color information. This allows black and white receivers to display NTSC signals simply by ignoring the chrominance. In NTSC, chrominance is encoded using two 3.579545 MHz signals that are 90 degrees out of phase, known as I (in-phase) and Q (quadrature) QAM. Mathematically, the combination of two sine waves 90 degrees out of phase with each other, with varying respective amplitudes, can be viewed as a single sine wave with varying phase relative to a reference, and varying amplitude. In essence, the phase represents the instantaneous color hue captured by a TV camera and the amplitude represents the color saturation.

For a TV or a display to recover hue information from the I/Q phase as just described, it must know the reference for it (i.e. what phase is zero). It also needs a reference against which to compare the amplitude to make saturation sense out of it. So the NTSC signal includes a short sample of this reference signal, known as the color burst, located on the 'back porch' of each horizontal line (the time between the end of the horizontal synchronization pulse and of the blanking pulse on each line). The color burst consists of a minimum of eight cycles of the unmodulated (fixed phase and amplitude) color subcarrier. By comparing the reference signal derived from color burst to the chrominance signal's amplitude and phase at a particular point in the scan, the device determines what chrominance to assign to the pixel then being displayed. Combining that with the amplitude of the luminance signal, the receiver calculates exactly what color to make the pixel.

When a transmitter broadcasts an NTSC signal, it amplitude-modulates a radio-frequency carrier with the NTSC signal just described, while it frequency-modulates a carrier 4.5 MHz higher with the audio signal. If non-linear distortion happens to the broadcast signal, the 3.58 MHz color carrier may beat with the sound carrier to produce a dot pattern on the screen. To make the resulting pattern less noticeable, designers adjusted the original 60 Hz field rate down by a factor of 1000/1001, to approximately 59.94 fields per second.

The 59.94 rate is derived from the following calculations. Designers chose to make the chrominance subcarrier frequency an n + 0.5 multiple of the line frequency to minimize interference between the luminance signal and the chrominance signal. They then chose to make the audio subcarrier frequency an integer multiple of the line frequency to minimize interference between the audio signal and the chrominance signal. The original black and white standard, with its 15750 Hz line frequency and 4.5 MHz audio subcarrier, does not meet these requirements, so designers had either to raise the audio subcarrier frequency or lower the line frequency. Raising the audio subcarrier frequency would prevent existing receivers from properly tuning in the audio signal. Lowering the line frequency is comparatively innocuous, because the horizontal and vertical synchronization information in the NTSC signal allows a receiver to tolerate a substantial amount of slop in the line frequency. So that is the route the color standard took. In the black and white standard, the ratio of audio subcarrier frequency to line frequency is 4.5 MHz / 15750 = 285.71. In the color standard, this becomes rounded to the integer 286, which means the color standard's line rate is 4.5 MHz / 286 ~ 15734 lines per second. Dividing by 262.5 lines per field, this gives approximately 59.94 fields per second.