BOT Instructor’s Board additions to add ATV option.

Install the 9-volt battery clips on the board at the indicated location.  Take special note of the clip gender orientation.  Use this illustration or look at the side of a 9-vole battery that has the polarity marked.  Soldering these components will require more heat than normal.

Install the camera and power interface connectors at the indicated location on the board.  Take note of the connector outline and make sure the orientation tab is located as indicated on the board.  Take note that the camera interface connector has the pins labeled: G = ground, V = Video, A = Audio.  You will need this information when you install the connector on the camera lead.  The power connector (2-pin) does not have the pins labeled.  Make sure you know which pin is connected to 9-volts and which pin is connected to ground.  It is vital that the connector to the power cable is wired with the correct polarity or you could damage the camera.

Install the reverse SMA connector for the ATV antenna from the component side of the board.  This connector will require more than the normal amount of heat.

Install the BNC connector on the bottom side of the board and solder the leads from the top side of the board.  Mount the ATV-TX interface connectors from the underside of the board.  The components are outlined on the top side of the board but the actual components are mounted on the bottom of the board.  Though the interface and power connector pins are not marked on the board, the pin-outs are the same as the camera connectors.  It is vital that you take note of the pin-outs of these connectors when you install the companion connectors on the ATV-TX module or you could damage the module.

ATV Camera interface connector installation.   Note that the stock connector plugs are not used during the installation on the BOT.  The black plug is the power plug.  Clip off the black plug and you will see a red wire (the + side) and a stranded bare wire (the ground side).  Install the appropriate connector to these wires.  Make sure that when you insert these pins into the connector housing that you observe the proper polarity.  Check it a couple of times, install the connector onto the BOT board without power attached and trace the wires with a VOM to make double sure.  Clip off the yellow (video) and while (audio) plugs.  Twist the bare stranded copper wires together (ground).  Install the appropriate connector to these wires.  Likewise make sure that when you insert these pins into the connector housing that you observe the proper pin-outs.  Double check the connections with a VOM while the board is not powered.

ATV-TX interface connector installation.  You will have to install a male BNC connector on the TX coax line.  Take some time to made a good quality installation of the connector.  It will prevent problems in the future.  Note the power leads do not have any connector.  Install the appropriate connector to the power leads making sure you observe the right polarity for the companion connector on the BOT board.  Use a VOM to trace the connections while the board is not powered.

Check with the documentation on the TX to identify the color of the video and the audio lines, make a tape label for each wire before you clip off the color coded connector.  Twist the braded copper ground wires together.  Install the appropriate connector and make sure that you keep the pin-outs straight with the connector on the board.  Use a VOM to trace the wires while the board is not powered double check the connector installation.

The camera is mounted on a standard servo, the one that came with the What is a Micro Controller course (it turns only 180 degrees).  Attach the camera mounting bracket on the servo cam with machine screws.  The servo will be connected to pin 14 of the BOT circuit board via the appropriate servo connector.

Double check that all the connectors are installed with the proper polarity and pin-outs using a VOM to trace the leads before you apply power.  The BOT is front heavy with the camera installed, so it is helpful to have batteries installed when operating the camera configured BOT.  Screw the ATV antenna onto the SMA connector before operating the transmitter.  I installed the ATV-TX under the BOT main circuit board, it is a tight fit so there is no need for any adhesive to keep it in place.  Just make the channel selection before you install the TX because you can’t change it without taking things apart again.  The camera servo is controlled by TV remote volume and channel controls, see the program for details.   During the Explore simulation, the camera is fixed.  You can modify the program if you want the camera to play a more active role in your simulation. The ATV system eats batteries, so don’t expect the 9-volt batteries to last for very long.  You can probably get some time with just one battery, but the more the better.  I actually modified an HT battery to clip onto the BOT to power the ATV to get longer life.