With the increasing importance of digital communications via Amateur Radio, this project should be of interest to many hams.  The Logic Analyzer described in this article should meet the needs of most home experimenters.  The hardware design is such that the individual implementor may make changes and still use the software I have developed.  The software is available as “Shareware”.  A free version is available which uses no hardware other than your printer port (EPP mode).

This Logic Analyzer is very similar to the earlier commercial instruments.  It takes a sample every clock cycle and stores the data for display.  The memory depth defines the number of samples as well as the total sample interval.  In this case the memory size is 131K; therefore, with a sample period of 200ns the sample interval will be approximately 26 milliseconds.

Newer Logic Analyzers have a much faster sample clock (usually pico-seconds) and instead of storing data at each sample period they store data whenever a state change is detected.  The time of the change is stored along with the data.  This allows a greater degree of flexibility in the analysis of the data but may also require more memory.  Some mechanism must be implemented which will store the time of each state change.  This could require a separate memory device.

Some of the general specifications are:

- 8 channels

- 5volt logic only

- time resolution from 200ns to 1ms per sample

- Windows 3.1 or Windows 95

- Enhanced Parallel Port required

- user configuration via INI file

- pre-trigger capability of 4096 (2^12) samples

- memory depth of 131K (2^17) bytes

- circuits built with HCMOS parts - usually good to at least 25 MHz.

The operation of each circuit is described so that the reader might gain some insight as to how they work as well as ideas for modification.  A few possible suggestions for modifications are:

- Increase clock speed to 20 MHz to allow 50ns sampling

- Change the pre-trigger memory size

- Allow more sampling rate choices - including an external input

A few notes on the circuit descriptions:

- The suffix characters of the signal names have the following meanings:

PL = Pulse Low: the active level is a pulse which has a steady state

value of 5v but pulses low to 0v for a “short” period of time

PH = Pulse High: similar to above, but pulses high

L = active level is 0v

H = active level is 5v

2. Timing values are usually indicated with Hex notation: 0x....

I use this because I do a lot of programming in C

3. Only those signals which go between the circuits have been named

The strip menu has a Debug entry which allows the builder to test the system.  I did most of my hardware debugging using this facility.  Several of the circuits can be tested as they are built.  The ICs do not all have to be installed before testing begins.  A suggested order is: Control, Time Base, Address Generator, Input, Trigger, RAM, and Pre-Trigger.  The Time Base and Address Generator can be built nd tested independantly of the Control circuit if desired.

The Help menu is not available at the time of writing this article.  I hope to have it ready by the time this goes to print.  It will contain much of the operational information in this article as well as details on the Debug options.

I am very interested in any feedback which anyone may have about the operation of this system.  I will try to incorporate any changes which I feel enhance its operation.

Circuits (in alphabetical order) Theory of Operation

Address Generator (built on “daughter board”)

This circuit is constructed using synchronous binary counters.  As such they generate a binary sequence creating the address information for both the RAM IC and the Pre-Trigger address.  The advantage of a synchronous counter is that all the outputs change state using the same clock edge.  This insures that there is no delay stack-up from the least to the most significant stages.

The operation of the circuit is as follows:

Until a trigger signal occurs, U404 & U405 are disabled, thus allowing U401 - U403 to form the address counter for the pre-trigger memory.  They (U401-U403) start counting immediately following the AddrReset-PL signal.  They will continue to count until they reach 0xFFF (4095.), at which point they will roll-over and begin again at 0x000.  This will continue until a trigger signal occurs.  At that time both the Triggered-PL and the Triggered-H signals will occur.  The Triggered-PL will preset the address counter to a value of 0x21000.  Please note two significant facts about this value:

1) the 0x20000, bit 17, is NOT used as an address bit

2) the address is one count beyond the pre-trigger range

The Triggered-H (via HighAddrEna-H) signal enables the last two ICs in the circuit.  Eventually the count will reach 0x3FFFF and then roll-over to 0x40000.  This will inhibit any further counting because the AddrDone-L signal, derived from bit 17, is fed back to one of the enable inputs of U401, the first IC in the counter chain.  The AddrDone-H signal is used to tell the PC that all the samples have been stored.

When the system is sampling the input data, the AddrClk-PH signal is derived from the Time Base circuit.  When the data needs to be dumped to the PC, the signal is derived from the PC via one of the control port bits.

Note that the same signal is used to advance the counter as well store the data value into the RAM.  This is valid because the RAM chip does not require any “data hold” time in order to store the data.  Therefore, by the time the address lines have changed (spec = 30ns, typical), the RAM is done with the data for the address just ended.

When the system is not sampling input data the clock signal for the counter chain is derived from the “Strobe”.  The AddrReset-PL signal is delayed by an R-C delay circuit in order to insure that the “Load” signal of U405 is still low when the clock signal goes high.  This is required because the 74HC161 has a synchronous load circuit, meaning that the counter performs its load operation while the load input is low when the clock goes high.

Control

This section contains severl different functions:

1) U104 is used to select whether the PC needs to read from or write to the system.  It generates four Read Enables, only 3 of which are used, and the four Write Pulses which are used to latch control data from the PC.

2) U101 and U102 latch the LA control signals: time base select, sample enable, and address reset.

3) U105 controls the clock signals for both the address generator and the RAM as well as the count enable for U404 and U405.

4) U106A allows triggering to begin.  This insures that the trigger circuit will not be enabled until the pre-trigger RAM is full.

Input

The 74HC14 is a Schmitt Trigger.  As such, it gives the LA some degree of noise immunity from the input signals.  These signals are delivered both to the RAM and the trigger circuits.  Since the RAM data ports are bi-directional, this circuit also makes the RAM data available to the PC.

Please note that this input circuit is valid for 5 volt logic only!  If you have a need for any other logic levels you MUST condition the signals before applying them to this circuit or the ICs may be damaged.  The input resistors (1K) give some degree of protection since there are diodes within the 74HC14 both to ground and power.  The resistors are placed close to the input pins in order to minimize the effect of input (spec = 10pf, max) and wiring capacitance.  There is also a set of “pull-down” resistors used to insure that the HCMOS inputs are never floating.

Pre-Trigger

When a trigger signal occurs the value of the next address will be stored in the Pre-Trigger Address latch by the Triggered-H signal.  This value is made available to the LA program so that it can determine the address in the range 0x00000 to 0x00FFF (0 - 4095) at which the trigger event occurred.  The program can then display the data values in the correct sequence for the pre-trigger display.

RAM

The RAM device was chosen mainly due to availability in a DIP. This device has a minimum clock pulse width requirement of 60ns.  This does not allow the system to operate at its fastest sample rate of 10MHz (100ns).  It will work at 5MHz (200ns) which is fast enough for my current requirements.  It would require a very simple modification to the circuit to accommodate another memory device which would have a faster response time.  It may be as simple as just plugging it in - with the signals routed appropriately.

When sampling the input data the SampleClk-PH signal is applied to the R/W line.  When reading the data back to the PC the R/W signal is a static level generated via the Control logic.

Time Base (built on “daughter board”)

When the system is sampling the input data, this circuit generates the sample clock used to advance the address counter as well as the signal which writes the data into the RAM.  Although only 3 bits are currently implemented to select the sample rate, there are 5 bits available.  This allows for up to 32 selections.  The INI file allows the user to define the sample period for each implemented selection.  The primary reason for allowing 2 more bits is so that multiplier values of x1, x2, or x5 may be implemented.  This will give sample periods which are similar to those of an oscilloscope.

Trigger (built on “daughter board”)

The trigger circuit is used to generate the signals which enable the system to begin storing the input data beyond the Pre-Trigger address range.  It allows the user to specify the trigger state for each of the 8 input lines individually.  Any one of three values is allowed: 0, 1, or “don’t care”.  The operation is such that when the system is enabled by the user, the address generator will generate only those addresses which are defined as the “pre-trigger” range.  When the trigger event occurs the address generator will still increment to the next address.  This address is stored in the pre-trigger latch circuit.  The trigger event will always be displayed at sample 4095.  The next sample will be stored at address 0x01000 (4096).  See the timing diagram.

The trigger state selection is accomplished using 2 control bits for each input.  The least significant bit (bit 0) is used to define the trigger input level as being either a “1” or a “0”.  The most significant bit (bit 1) is used to enable or disable the input signal from reaching the comparator.  Here is a truth table describing the operation:

Bit Trigger

1 0  State

0 0	always

0 1	never

1 0	  0

1 1	  1

Please note that the triger circuit is synchronous.  This means that the trigger event will only be detected at the rising edge of the sample clock.

Some Construction Information

In general construction details have not been discussed since the individual implementor may choose among several methods.  Some general information on the circuit as I built it:

1. Three of the circuits were built on “daughter” boards which plug into the main board via pin strip headers and sockets.  This method was chosen mainly for two reasons:

a. The largest circuit board on hand was not large enough to hold all the Ics.  The one in the parts list would also require the use of “daughter” boards.  Larger ones are available from the same vendor.

b. Those circuits which are on plug-in boards can be easily replaced in order to experiment with modified designs.

2. Several construction details have been deliberatly omitted, such as: power supply, cable to the PC, probe cable, mounting methods, etc.  This was done mainly in order to allow the builder to determine the best methods for his/her needs.

Operation of the Logic Analyzer front panel controls

Starting Sample Number

Position of the desired sample number at the left hand edge of the display window

Ending Sample Number

Position of the desired sample number at the right hand edge of the display window

Note: the number of samples viewed at any one time is limited to the range 50 <= number of samples <= 30000

Cursor

Move the cursor to the desired sample.  The cursor may also by using the mouse to “click” on a location within the display area.

The above controls allow the user to type the desired number into the appropriate input box.  However, the entered value does not take effect until the Enter key is pressed. This mode of operation was chosen in order to disallow what might be illegal values due to partial entries.  In order to give the operator some visual feedback the background color of the box will change until the Enter key is pressed.

Slide Up

Move the display window to the set of samples which begin at 90% of the ending sample number, keeping the same number of samples in the display window

Slide Down

Move the display window to the set of samples which begin at 10% of the ending sample number, keeping the same number of samples in the display window

Cursor Memory Check Boxes

these boxes allow the user to store the current cursor location.  As the cursor is moved the “Distance to Cursor” values are updated.  this allows the user to make time difference measurements.

Draw

Under most circumstances the program knows when to draw the waveforms.  However, the waveforms will develop “holes” when the cursor is moved.  This control is then used to re-draw the waveforms.  I do not currently know of any method within Visual Basic which will allow the cursor to move within the display window without leaving these “holes”.

Warning: the time to display the window is directly proportional to the number of active channels and the number of samples to be displayed.  It can take several seconds to draw one signal if there are 30000 samples.  This is processor dependant - I currently have a 166MHz ‘686.

Trigger

This signals the hardware to acquire another sample set.

Notes on the free version of the program:

- Only 2 channels are enabled: D0 and D1 of the printer port

- No external hardware is supported

- Triggering will occur after the Trigger control has been pressed and a change is detected in either or both signals.

Usage of the Printer Port, in EPP mode, for the Logic Analyzer:

Data Port (read and write): Base + 0

bits 0 .. 7: PPDR-Bit0 .. 7 = pins 2 .. 9

Status Port (read only): Base + 1

bit 6: AddressDone-H = pin 10

bit 7: Triggered-L = pin 11

Control Port (write only): Base + 2

bit 0: strobe = pin 1

bit 1: RegSel-0 = pin 14

bit 2: RegSel-1 = pin 16

bit 3: RegSel-2 = pin 17

bit 5: Enable bi-directional port (0=write,1=read) - this signal is NOT available on the Printer Port connector.

The design implements 2 sets of hardware registers which allow the Data Port to be used both to read and write based on the Register Select bits:

Register 0 Write: Control

bit 0: SampleEna-L, SampleEna-H

bit 1: AddrReset-PL

Register 1 Write:

TimeBaseSel-0 .. TimeBaseSel-4

Register 2 Write:

TrigSel-00 .. TrigSel-31

Register 3 Write:

TrigSel-40 .. TrigSel-71

Register 4 Read: not used

Register 5 Read:

ReadReg5-L:

stored data from RAM

( strobe is used to advance the address )

Register 6 Read:

ReadReg6-L:

low 8 bits of Pre-Trigger address

Register 7 Read:

ReadReg7-L:

bits 0..3: upper 4 bits of Pre-Trigger address

Screen dump of the Logic Analyzer main screen

Trigger Circuit Timing Diagram

The symbols used in the diagram:

_ is 0 volts,	= is 5 volts,	X is “don’t care”,

.. indicates time duration

Addr Count	  	0   1		   4095			 N >< PreTrigCount >< 4096

SampleClk		_..___========__....__========__....__========________===

SampleEna-H		_____====================================================

AddrReset-PL	==_======================================================

TrigStart-H		________________________=================================

Trigger Event	XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX____XXXXXXXXXXXXXXXXX

Triggered-PL	=======================================________==========

Triggered-H		________________________________________=================

Parts List

Vndr: M = Mouser 	RS = Radio Shack	DK = DigiKey	J = Jameco

Part			    Qty  Vndr	Part Number		Price Ea	Ext Price

74HC126			2	M	511-M74HC126		$0.46 	$0.92

74HC14			3	M	511-M74HC14			$0.46 	$1.38

74HC74			1	M	511-M74HC74			$0.46 	$0.46

74HC151			1	M	511-M74HC151		$0.51 	$0.51

74HC155			1	M	511-M74HC155		$0.55 	$0.55

74HC157			1	M	511-M74HC157		$0.51 	$0.51

74HC161			5	M	511-M74HC161		$0.60 	$3.00

74HC4518			2	M	511-M74HC4518		$0.71 	$1.42

74HC273			6	M	511-M74HC273		$0.71 	$4.26

74HC540			1	M	511-M74HC540		$0.81 	$0.81

74HC541			3	M	511-M74HC541		$0.81 	$2.43

74HC688			1	M	511-M74HC688		$0.94 	$0.94

8 pin socket		7	M	571-26403573		$0.04 	$0.28

14 pin socket		1	M	571-26404633		$0.06 	$0.06

16 pin socket		10	M	571-26403583		$0.07 	$0.70

20 pin socket		11	M	571-26404643		$0.10 	$1.10

32 pin socket		1	M	571-26440183		$0.19 	$0.19

47uf, 10v uf cap.		4	M	140-XRL10V47		$0.07 	$0.28

0.2uf cap.			20	M	21RZ320			$0.17 	$3.40

Resistor Net, 47K		1	M	266-47K			$0.25 	$0.25

pin strip header,		3	M	517-5111TN			$0.78 	$2.34

rt angle

PC Board, 4.5 x 8"	1	M	574-3677-6			$16.81 	$16.81

25 Pin D sub,		1	M	571-7479122			$3.72 	$3.72

Male chassis

9 Pin D sub,		1	M	156-1309			$0.66 	$0.66

Fem cable

pin strip header,		1	J	SMH36				$0.49 	$0.49

straight

Housing, 36 pin		3	J	SCH36				$0.59 	$1.77

Pins for SCH36		100	J	FCH1				$0.06 	$6.00

Sw, DPDT-2P			1	J	21979				$1.09 	$1.09

PC Board			3	RS	276-168B			$2.49 	$7.47

50’Wrapping Wire		1	RS	278-501			$2.99 	$2.99

9 pin D Sub hood		1	RS	276-1539			$0.99 	$0.99

Resistors, Assorted	1	RS	271-312			$7.99		$7.99

9 Pin D sub,		1	DK	A2043-ND			$1.89 	$1.89

chassis male

Box: Alum,			1	DK	L108-ND			$16.62 	$16.62

10 x 6 x 3.5

Test Clips,			2	DK	923848			$7.88 	$15.76

set of 6

TC551001BPI			1	DK	TC551001BPL-70-ND		$6.75 	$6.75

(1MB RAM)

10MHz Osc			1	DK	SE1208-ND			$3.15 	$3.15

Total = 	$119.94

Notes on the BOM:

1) usage of the headers, housings, and PW boards are dependent on the construction techniques used by the builder.

2) many of the parts can be obtained from several vendors